Andrew Rutherford

CSCI 4113

Lab 6

- On machine F, ‘ip addr’ to get machine F’s mac address: 00:50:56:b4:c9:89

- ‘vi /etc/sysconfig/network-scripts/ifcfg-eno16780032’ and set ONBOOT=YES

- On machine A, add machine F mac address, and fixed IP address in /etc/dhcp/dhcpd.conf

subnet 100.64.36.0 netmask 255.255.255.0 {

authoritative;

range 100.64.36.0 100.64.36.254;

default-lease-time 3600;

max-lease-time 3600;

option subnet-mask 255.255.255.0;

option broadcast-address 100.64.36.255;

option routers 100.64.36.0;

option domain-name-servers 8.8.8.8;

option domain-name "example.com";

}

host machine_F {

hardware ethernet 00:50:56:b4:c9:89;

fixed-address 100.64.36.5;

option host-name machineF;

}

- Machine F can now be SSH’ed into, but has no internet access.  To fix this set default gateway: ‘route add default gw 100.64.36.1’

- On machine f:

  - Yum install httpd

  - Systemctl enable httpd

  - Systemctl start httpd

- ‘yum install rsync’ on machine F and machine B

- On machine B, rsync /etc/sudoers,group,shadow,passwd to machine F.

- On machine F, edit crontab: ‘crontab –e’
0 * * * * rsync –chavzP –stats root@100.64.36.2:/var/www/ /var/www/

- Set up rsync to not require password every time:

  - On machine F: ‘ssh-keygen’, ENTER, ENTER, ENTER

  - ‘ssh-copy-id –I ~/.ssh/id_rsa.pub 100.64.36.2’

- Web browser at 100.64.36.5 still shows apache test page.

  - Cp –r /var/www/dundermifflin/* /var/www/html/